Log Analysis.

Author: Akshata Jadhav

Designation: Project Engineer

Date: 20th july 2019

REVISION HISTORY

| VERSION | DATE | Summary Of Changes | AUTHOR | REVIEWED BY | RIVISION REMARKS (YES / NO) |
| --- | --- | --- | --- | --- | --- |
| 0.1 | 20st June, 2019 | Initial Draft | Akshata Jadhav |  |  |
| 0.2 | 20th July 2019 | Modifications in script | Akshata Jadhav |  |  |
| 0.3 |  |  |  |  |  |

Contents

Introduction	4

Flow Chart	4

Process	5

Execution Steps	5

Input Parameter:	5

Output	5

Scope	5

Prerequisites	5

Deployment Steps / Procedure	6

# Introduction

The script will Login to core Switch, EOR Switch and TOR switch. Triggers commands and get output in text file.

# Flow Chart

# Process

When the script is triggered it will Login to core Switch and trigger a set of commands to get hostname of EOR switch. Then the Script logins to EOR switch and triggers a set of commands to get hostname of TOR switch. Then the script logins to TOR switch Triggers commands. Output of all commands  is saved in text file.

# Execution Steps

- Script login to Core Switch and triggers following commands:

- Show arp <server ip>: Get mac address from output of this command

- Show mac address <mac address>: Get slot/interface from output of this command

- Show lag ethernet <slot/interface>: Get EOR hostname from output of this command

- Script Login to EOR Switch and triggers following commands:

- Show mac-address-table add <mac address>:Fetch the Rbridge ID from output of this command

- Show vcs : Fetch the host name TOR switch with Rbridge id from output of this command

- Script login to 1st TOR Switch triggers the following commands

  - Show interface Te slot/rbridge/port

  - Show logging raslog | slot/rbridge/port

- Checks the host name of TOR switch if the 1st switch host name includes TOR_A then the 2nd  switch hostname to be login and run the same set of commands will have TOR_Z.

# Input Parameter:

# CSV file with the  Core switch credentials encrypted

# Output

# Output for all the commands will be saved in a text file  with a file name Log_Analysis< time stamp.>.txt

# Scope

This script will work on python 3.4 and above version.

# Prerequisites

•Python 3.4 and above version

# Deployment Steps / Procedure

- Edit the input file with device credentials.

- Run the script  and the output text file gets generated.