# Log Analysis

# Core switch Log Analysis

- Login into Core Switch

- Ping < Server ip address >

- Show arp | i < server ip address>

Get mac add and enter in next commands

- Show mac-address HHHH.HHHH.HHH

Get slot/interface and enter in next command

- Show lag ethernet slot/interface

# EOR switch Log Analysis

- Login into EOR Access Switch

- Show mac-address-table add HHHH.HHHH.HHHH

- Show VCS

# TOR Switch Log Analysis

- Login into TOR Access Switch A

- Show Interface Te slot/rbridge/port

- Show logging raslog | I slot/rbridge/port

- Login into TOR Access Switch B

- Show Interface Te slot/rbridge/port

- Show logging raslog | I slot/rbridge/port

- Login into Splunk and search with switch and port.

- Login into Cacti and select switch and port.

Core Switch – Brocade MLX switch

Access Switch – Brocade VDX switch

from raj (privately):

Show arp | i < server ip address>

from raj (privately):

SSH@HOST#ping 10.115.3.46

Sending 1, 16-byte ICMP Echo to 10.115.3.46, timeout 5000 msec, TTL 64

Type Control-c to abort

Reply from 10.115.3.46     : bytes=16 time=33ms TTL=128

Success rate is 100 percent (1/1), round-trip min/avg/max=33/33/33 ms.

SSH@HOST#sh arp | i 10.115.3.46

1     10.115.3.46         0050.568c.d7a9      Dynamic     0        4/1

^C

SSH@HOST#sh mac

mac-address                   MAC address table

SSH@HOST#sh mac-address 0050.568c.d7a9

Total active entries from all ports = 3667

Type Code - ST:Static SEC:Secure 1x:Dot1x NA: NotAvail A:Allow D:Deny

CCL: Cluster Client Local CCR:Cluster Client Remote CL:Local CR:Remote

MAC Address     Port      Age      VLAN    Type

0050.568c.d7a9   4/3         0       250    CCR

SSH@HOST#sh lag eth

ethernet       Ethernet port

SSH@HOST#sh lag ethernet 4/3

Total number of LAGs    : 15, 100/40g : 0

Total number of deployed LAGs   : 15, 100/40g : 0

Total

from raj (privately):

EOR output

from raj (privately):

HOST_A103# show mac-address-table add 0050.568c.d7a9

VlanId   Mac-address       Type     State        Ports

250      0050.568c.d7a9    Dynamic  Remote       Te 7/0/9

Total MAC addresses    :  1

HOST_A103# sh vcs

Config Mode    : Local-Only

VCS ID         : 5

Total Number of Nodes           : 10

Rbridge-Id       WWN                            Management IP   Status          HostName

----------------------------------------------------------------------------------------

3                10:00:50:EB:1A:07:BE:C5        xxx.xxx.xxx.123  Online          HOSTR_A099

4               >10:00:50:EB:1A:10:D4:C1        xxx.xxx.xxx.127  Online          HOSTR_Z099

7                10:00:50:EB:1A:10:CB:09        xxx.xxx.xxx.124  Online          HOSTR_A100

8                10:00:50:EB:1A:10:D4:41        xxx.xxx.xxx.128  Online          HOSTR_Z100

11               10:00:00:27:F8:91:DE:98*       xxx.xxx.xxx.121  Online          HOST_A103

15               10:00:00:

SSH@HOST#sh lag ethernet 4/1
Total number of LAGs    : 15, 100/40g : 0
Total number of deployed LAGs   : 15, 100/40g : 0
Total number of trunks created  : 15 (113 available), 100/40g : 0 (8 available)
LACP System Priority / ID       :1 / 748e.f8fc.ce00
LACP Long timeout       :90, default: 90
LACP Short timeout      :3, default: 3
=== LAG "VCS5_vLAG" ID 11 (dynamic Deployed) ===
LAG Configuration:
   Ports:         e 4/1 to 4/8
   Port Count:    8
   Primary Port:  4/1
   Trunk Type:    hash-based
   LACP Key:      110
   LACP Timeout:  short
Port Individual Configuration:
   Port Name
   4/1  HOST_A103 x/5
   4/2  HOST_A103 x/10
   4/3  HOST_A103 x/15
   4/4  HOST_A103 x/20
   4/5  HOST_Z103 x/25
   4/6  HOST_Z103 x/30
   4/7  HOST_Z103 x/35
   4/8  HOST_Z103 x/40

show lag ethernet slot/interface | i EOR

SSH@HOST#sh lag ethernet 4/1 | i EOR
   4/1  HOST_A103 x/5
   4/2  HOST_A103 x/10
   4/3  HOST_A103 x/15
   4/4  HOST_A103 x/20
   4/5  HOST_Z103 x/25
   4/6  HOST_Z103 x/30
   4/7  HOST_Z103 x/35
   4/8  HOST_Z103 x/40

from Raj to everyone:

HOST_A103

from Raj to everyone:

SSH@HOST#sh lag ethernet 4/1 | i EOR
   4/1  HOST_A103 x/5
   4/2  HOST_A103 x/10
   4/3  HOST_A103 x/15
   4/4  HOST_A103 x/20
   4/5  HOST_Z103 x/25
   4/6  HOST_Z103 x/30
   4/7  HOST_Z103 x/35
   4/8  HOST_Z103 x/40

HOST_A103# show mac-address-table address 0050.568c.d7a9
VlanId   Mac-address       Type     State        Ports
250      0050.568c.d7a9    Dynamic  Remote       Te 23/0/9
Total MAC addresses    :  1

HOST_A103# sh vcs | i 23
3                10:00:50:EB:1A:07:BE:C5        xxx.xxx.xxx.123  Online          HOSTR_A099
23               10:00:50:EB:1A:10:CA:09        xxx.xxx.xxx.126  Online          HOSTR_A102

"